Multicast SENDER

#include <sys/types.h>   /* for type definitions */

#include <sys/socket.h>  /* for socket API function calls */

#include <netinet/in.h>  /* for address structs */

#include <arpa/inet.h>   /* for sockaddr_in */

#include <stdio.h>       /* for printf() */

#include <stdlib.h>      /* for atoi() */

#include <string.h>      /* for strlen() */

#include <unistd.h>      /* for close() */

#define MAX_LEN  1024    /* maximum string size to send */

#define MIN_PORT 1024    /* minimum port allowed */

#define MAX_PORT 65535   /* maximum port allowed */

int main(int argc, char *argv[]) {

int sock;                   /* socket descriptor */

char send_str[MAX_LEN];     /* string to send */

struct sockaddr_in mc_addr; /* socket address structure */

unsigned int send_len;      /* length of string to send */

char* mc_addr_str;          /* multicast IP address */

unsigned short mc_port;     /* multicast port */

unsigned char mc_ttl=1;     /* time to live (hop count) */

/* validate number of arguments */

if (argc != 3) {

fprintf(stderr,

"Usage: %s <Multicast IP> <Multicast Port>\n",

argv[0]);

exit(1);

}

mc_addr_str = argv[1];       /* arg 1: multicast IP address */

mc_port     = atoi(argv[2]); /* arg 2: multicast port number */

/* validate the port range */

if ((mc_port < MIN_PORT) || (mc_port > MAX_PORT)) {

fprintf(stderr, "Invalid port number argument %d.\n",

mc_port);

fprintf(stderr, "Valid range is between %d and %d.\n",

MIN_PORT, MAX_PORT);

exit(1);

}

/* create a socket for sending to the multicast address */

if ((sock = socket(PF_INET, SOCK_DGRAM, IPPROTO_UDP)) < 0) {

perror("socket() failed");

exit(1);

}

/* set the TTL (time to live/hop count) for the send */

if ((setsockopt(sock, IPPROTO_IP, IP_MULTICAST_TTL,

(void*) &mc_ttl, sizeof(mc_ttl))) < 0) {

perror("setsockopt() failed");

exit(1);

}

/* construct a multicast address structure */

memset(&mc_addr, 0, sizeof(mc_addr));

mc_addr.sin_family      = AF_INET;

mc_addr.sin_addr.s_addr = inet_addr(mc_addr_str);

mc_addr.sin_port        = htons(mc_port);

printf("Begin typing (return to send, ctrl-C to quit):\n");

/* clear send buffer */

memset(send_str, 0, sizeof(send_str));

while (fgets(send_str, MAX_LEN, stdin)) {

send_len = strlen(send_str);

/* send string to multicast address */

if ((sendto(sock, send_str, send_len, 0,

(struct sockaddr *) &mc_addr,

sizeof(mc_addr))) != send_len) {

perror("sendto() sent incorrect number of bytes");

exit(1);

}

/* clear send buffer */

memset(send_str, 0, sizeof(send_str));

}

close(sock);

exit(0);

}

Multicast RECEIVER

#include <sys/types.h>  /* for type definitions */

#include <sys/socket.h> /* for socket API calls */

#include <netinet/in.h> /* for address structs */

#include <arpa/inet.h>  /* for sockaddr_in */

#include <stdio.h>      /* for printf() and fprintf() */

#include <stdlib.h>     /* for atoi() */

#include <string.h>     /* for strlen() */

#include <unistd.h>     /* for close() */

#define MAX_LEN  1024   /* maximum receive string size */

#define MIN_PORT 1024   /* minimum port allowed */

#define MAX_PORT 65535  /* maximum port allowed */

int main(int argc, char *argv[]) {

int sock;                     /* socket descriptor */

int flag_on = 1;              /* socket option flag */

struct sockaddr_in mc_addr;   /* socket address structure */

char recv_str[MAX_LEN+1];     /* buffer to receive string */

int recv_len;                 /* length of string received */

struct ip_mreq mc_req;        /* multicast request structure */

char* mc_addr_str;            /* multicast IP address */

unsigned short mc_port;       /* multicast port */

struct sockaddr_in from_addr; /* packet source */

unsigned int from_len;        /* source addr length */

/* validate number of arguments */

if (argc != 3) {

fprintf(stderr,

"Usage: %s <Multicast IP> <Multicast Port>\n",

argv[0]);

exit(1);

}

mc_addr_str = argv[1];      /* arg 1: multicast ip address */

mc_port = atoi(argv[2]);    /* arg 2: multicast port number */

/* validate the port range */

if ((mc_port < MIN_PORT) || (mc_port > MAX_PORT)) {

fprintf(stderr, "Invalid port number argument %d.\n",

mc_port);

fprintf(stderr, "Valid range is between %d and %d.\n",

MIN_PORT, MAX_PORT);

exit(1);

}

/* create socket to join multicast group on */

if ((sock = socket(PF_INET, SOCK_DGRAM, IPPROTO_UDP)) < 0) {

perror("socket() failed");

exit(1);

}

/* set reuse port to on to allow multiple binds per host */

if ((setsockopt(sock, SOL_SOCKET, SO_REUSEADDR, &flag_on,

sizeof(flag_on))) < 0) {

perror("setsockopt() failed");

exit(1);

}

/* construct a multicast address structure */

memset(&mc_addr, 0, sizeof(mc_addr));

mc_addr.sin_family      = AF_INET;

mc_addr.sin_addr.s_addr = htonl(INADDR_ANY);

mc_addr.sin_port        = htons(mc_port);

/* bind to multicast address to socket */

if ((bind(sock, (struct sockaddr *) &mc_addr,

sizeof(mc_addr))) < 0) {

perror("bind() failed");

exit(1);

}

/* construct an IGMP join request structure */

mc_req.imr_multiaddr.s_addr = inet_addr(mc_addr_str);

mc_req.imr_interface.s_addr = htonl(INADDR_ANY);

/* send an ADD MEMBERSHIP message via setsockopt */

if ((setsockopt(sock, IPPROTO_IP, IP_ADD_MEMBERSHIP,

(void*) &mc_req, sizeof(mc_req))) < 0) {

perror("setsockopt() failed");

exit(1);

}

for (;;) {          /* loop forever */

/* clear the receive buffers & structs */

memset(recv_str, 0, sizeof(recv_str));

from_len = sizeof(from_addr);

memset(&from_addr, 0, from_len);

/* block waiting to receive a packet */

if ((recv_len = recvfrom(sock, recv_str, MAX_LEN, 0,

(struct sockaddr*)&from_addr, &from_len)) < 0) {

perror("recvfrom() failed");

exit(1);

}

/* output received string */

printf("Received %d bytes from %s: ", recv_len,

inet_ntoa(from_addr.sin_addr));

printf("%s", recv_str);

}

/* send a DROP MEMBERSHIP message via setsockopt */

if ((setsockopt(sock, IPPROTO_IP, IP_DROP_MEMBERSHIP,

(void*) &mc_req, sizeof(mc_req))) < 0) {

perror("setsockopt() failed");

exit(1);

}

close(sock);

}